Supplementary material

Methods Appendix 1:

The patient was positioned on the catheterization laboratory table and secured to a premounted supine cycle ergometer (Lode Angio, Lode, Groningen; eFigure 1). The ergometer was connected to a laptop computer with software (Lode Export Manager 10, V 10.5.1, Lode, Groningen) to initiate the exercise protocol and acquire performance data. The target-vessel was intubated with a standard 6F guide catheter from the right radial artery. Intra-arterial unfractionated heparin (70-100 U/kg) and intracoronary nitroglycerin (300mcg) were given prior to coronary angiography and physiological measurements.

The optimal working view was determined, and a standard coronary guidewire was advanced distally to secure the target vessel. A dual pressure and velocity sensor 0.014-in intracoronary wire (Combowire XT, Volcano Corp, California) connected to a stand-alone console (Combomap, Volcano Corp, California; eFigure 1) was then advanced to the tip of the guiding catheter and the pressure signals normalized. The Combowire tip-mounted sensor was advanced distal to the stenosis by a minimum of 15mm and its position recorded cineographically. An optimal Doppler velocity trace was obtained by rotational manipulation of the Combowire. Continuous pressure-flow measurements were performed under resting conditions, during a 2-minute intravenous infusion of adenosine and during an incremental exercise protocol. The order of adenosine and exercise was randomly assigned. A return to baseline hemodynamic conditions was mandated between each stage of the experimental protocol.

Prior to removal from the patient, the Combowire was returned to the catheter tip to assess for pressure drift. Stenting was then performed according to standard clinical practice. Stent optimization was performed at the operator’s discretion. Following stenting, the Combowire was reintroduced, advanced to the guiding catheter tip and renormalized as before. The Combowire was advanced to the same intracoronary position as previous, with cross-reference to the cine-acquired roadmap image for confirmation. All aforementioned stages of the pre-PCI study protocol were then repeated, including the incremental exercise protocol. Schematic representation of the study protocol is summarized in eFigure 1.

# Exercise protocol

An incremental exercise protocol starting at 40 Watts and increasing by 20 Watts every minute was used for all patients. The guiding catheter was disengaged from the coronary ostium for the duration of exercise to prevent vessel trauma and to permit central aortic pressure waveform recording without damping. Exercise was continued until the development of rate-limiting angina symptoms (pre-PCI) or physical exhaustion (post-PCI).

# Data analysis

The electrocardiogram, pressure waveforms and coronary flow velocity signals were directly extracted from the digital archive of the device console (ComboMap, V 1.9, Volcano Corporation) for offline analysis. Exercise data were exported from the ergometer software package using a dedicated export manager (Lode Export Manager 10, V 10.5.1, Lode, Groningen).

eFigure 1

eFigure 1: Experimental apparatus and study protocol

(A): Experimental apparatus and coronary catheter laboratory set up. (B): Schematic representation of the study protocol.

eTable 1: Calculation of physiologic indices

Pa 	= 	Proximal (aortic) pressure (mmHg)

Pd 	= 	Distal (coronary) pressure (mmHg)

FFR	= 	Pd/Pa at whole-cycle during pharmacological hyperemia

iFR 	= 	Pd/Pa at baseline over iFR window

HSR	= 	Pa-Pd (mmHg) / Whole - cycle hyperemic flow velocity (cm/s)

CFR 	=	Whole-cycle hyperemic flow velocity / Whole-cycle baseline flow velocity

Pa indicates aortic pressure; Pd, distal coronary pressure; FFR, fractional flow reserve; iFR, instantaneous wave-Free Ratio; HSR, hyperemic stenosis resistance; CFR, coronary flow reserve

eTable 2: Baseline characteristics

| Demographics (n=23) |  |
| --- | --- |
| Age (years) | 60.6 (8.1) |
| Male | 21 (91%) |
| Diabetes | 1 (4%) |
| Hypertension | 14 (61%) |
| Hyperlipidaemia | 16 (70%) |
| History of smoking | 9 (39%) |
| Previous myocardial infarction | 3 (13%) |
| LVEF < 40% | 0 (0%) |
| CCS class |  |
| I | 1 (4%) |
| II | 9 (39%) |
| III | 13 (57%) |
| Baseline hemodynamics |  |
| Systolic blood pressure (mmHg) | 140 (18) |
| Heart rate (bpm) | 68 (12) |
| Medications |  |
| Aspirin | 23 (100%) |
| Clopidogrel | 23 (100%) |
| Beta-blockers | 16 (70%) |
| Statin | 22 (96%) |
| ACE-I/ARB | 17 (74%) |
| Nitrates | 6 (26%) |
| CCB | 9 (39%) |

PCI indicates percutaneous coronary intervention; LVEF, left ventricular ejection fraction; CCS, Canadian Cardiovascular Society; bpm, beats per minute; ACE-I, angiotensin converting enzyme inhibitor; ARB, angiotensin II receptor blocker; CCB, calcium channel blocker.

eTable 3: Per vessel anatomical and physiological stenosis characteristics

| Vessel | Proximal stenosis location | Diameter stenosis by QCA | FFR pre-PCI | iFR pre-PCI | HSR pre-PCI | CFR pre-PCI | FFR post-PCI | iFR post-PCI | HSR post-PCI | CFR post-PCI | FFR | iFR | HSR | CFR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LAD | 64% (9/14) | 72.2% (11.3) | 0.50 (0.43-0.73) | 0.49 (0.33-0.87) | 1.65 (0.79-3.38) | 1.43 (1.11-2.29) | 0.89 (0.84-0.91) | 0.94 (0.93-0.98) | 0.27 (0.13-0.38) | 2.73 (2.28-3.12) | 0.32 (0.18-0.42) | 0.33 (0.09-0.57) | -0.87 (-2.38- -0.44) | 1.41 (0.75-1.50) |
| Cx | 80% (4/5) | 74.5% (5.8) | 0.64 (0.54-0.80) | 0.79 (0.49-0.87) | 1.22 (0.84-1.61) | 2.12 (1.30-2.15) | 0.97 (0.93-1.01) | 1.00 (0.98-1.01) | 0.04 (0.00-0.25) | 2.77 (2.58-3.20) | 0.29 (0.20-0.37) | 0.14 (0.06-0.28) | -0.95 (-2.70- -0.55) | 0.70 (0.44-1.19) |
| RCA | 0% 0/4 | 84.5% (7.3) | 0.43 (0.29-0.60) | 0.61 (0.40-0.78) | 2.95 (1.88-5.27) | 1.23 (1.08-1.36) | 0.96 (0.94-0.97) | 1.00 (0.98-1.01) | 0.10 (0.09-0.11) | 2.73 (2.50-3.04) | 0.49 (0.36-0.62) | 0.37 (0.23-0.56) | -2.81 (-5.13--1.78) | 1.51 (1.14-1.96) |

Values are n, mean ± SD, median (IQR) or n (%). LAD indicates left anterior descending; Cx, circumflex; RCA, right coronary artery; QCA, quantitative coronary angiography.

eTable 4: Univariate linear regression of baseline characteristics and anatomical stenosis characteristics on angina-limited exercise time.

| Univariate predictors of ETangina |  |  |  |
| --- | --- | --- | --- |
| Variable | β coefficient | 95% CI | p value |
| Patient characteristics |  |  |  |
| Age | -2.27 | -6.5 – 1.94 | 0.27 |
| Male | 32.3 | -88.5 – 153 | 0.58 |
| Diabetes | -48.8 | -215 – 118 | 0.55 |
| Hypertension | -24.8 | -94.1 – 44.6 | 0.47 |
| Hyperlipidaemia | -19.3 | -93.3 – 54.7 | 0.59 |
| History of smoking | -13.4 | -83.4 – 56.6 | 0.70 |
| Previous myocardial infarction | -62.1 | -160 – 35.8 | 0.21 |
| Number of antianginal medications | -3.84 | -59.2 – 51.5 | 0.89 |
| Anatomical stenosis characteristics |  |  |  |
| Reference vessel diameter (mm) | 3.93 | -76.0 – 83.9 | 0.92 |
| Stenosis diameter (%) | -0.68 | -4.05 – 2.70 | 0.68 |
| Stenosis length (mm) | 0.93 | -8.56 – 9.28 | 0.93 |
| Physiological stenosis characteristics |  |  |  |
| FFR | 226 | 59.5 – 393 | 0.01 |
| iFR | 198 | 97.2 – 299 | <0.001 |
| Log HSR | -122 | -191 – -52.2 | <0.01 |
| CFR | 44.7 | 0.21 – 91.7 | 0.05 |

ETangina indicates the change in exercise time following PCI. All abbreviations as per eTable 1

eFigure 2

eFigure 2: The relationship between angina-limited exercise time and anatomical stenosis severity

Scatter plots of the relationships between angina-limited exercise time and percentage stenosis diameter (black) and stenosis length in millimetres (white).